- Description

IPS050*JA [kHz] series is the specific SPXO IC for achieving 32.768kHz output by divide. The power consumption of IPS050*JA [kHz] series is quite Low, and comparable with tuning fork solution.

- Features

- Power consumption	: 12μA typical (IPS0503NAJAq, VDD=3.3V, No Load, Ta=25℃)

- Divide function		: 1/512 and 1/1024

- Output frequency	            : 32.768kHz

- Operation temperature	: -40℃~125℃

- Power supply voltage	: 1.08V~3.63V

- Standby function		: Oscillation stop

- Output		: CMOS

- Small chip size		: 0.55mm × 0.54mm

- Frequency stability to VDD	: Within ±1.0ppm

- Part number rule

Under Development

- Series

| Part Number |  |  |  |  |  |  | Crystal Frequency f （MHz） | Divide | Output Frequency F0 (kHz) | Pad Layout | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IPS050 | 3 | N | 9 | J | A | X | 16.777 | 1/512 | 32.768 | Cross |  |
| IPS050 | 3 | N | A | J | A | X | 33.554 | 1/1024 |  |  |  |
| IPS050 | 3 | N | 9 | J | A | F | 16.777 | 1/512 | 32.768 | Flip |  |
| IPS050 | 3 | N | A | J | A | F | 33.554 | 1/1024 |  |  |  |

Please contact us for gray color models.

- Absolute Maximum Ratings

Unless otherwise stated, VSS=0V, Ta=25℃±2℃

| Parameter | Symbol | Condition | Ratings |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max | Unit |
| Supply Voltage | VDD |  | VSS-0.5 | 5.0 | V |
| Input Voltage | VIN | Except below | VSS-0.5 | VDD+0.5 | V |
|  |  | X1, X2 | VSS-0.5 | 2.0 |  |
| Output Voltage | VOUT |  | VSS-0.5 | VDD+0.5 | V |
| Output Current | IOUT |  |  | 25 | mA |
| Junction Temperature | Tj |  | -55 | 150 | ℃ |
| Storage Temperature | Tstg |  | -55 | 125 | ℃ |

- Recommended Operating Condition

Unless otherwise stated, VSS=0V, Ta=-40℃~125℃

| Parameter | Symbol | Condition | Min | Typ | Max | Unit | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Supply Voltage | VDD |  | 1.08 |  | 3.63 | V | VDD |
| Input Voltage | VIN |  | VSS |  | VDD | V | CE |
| Output Load Capacitance | CL | CMOS |  |  | 15 | pF | OUT |
| Ambient Temperature | Topt |  | -40 |  | 125 | ℃ |  |

- Electrical Specification

Unless otherwise stated, VDD=1.08V~3.63V, VSS=0V, CL=15pF, Ta=-40℃~125℃

| Parameter | Symbol | Condition |  |  | Specification |  |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Min | Typ | Max |  |
| Output leak current | Iz | CE=0V, X1=2.0V or VSS Vout=VSS~VDD |  |  |  |  | 20 | μA |
| “H” input voltage | VIH | CE Pad |  |  | 0.8VDD |  |  | V |
| “L” input voltage | VIL | CE Pad |  |  |  |  | 0.2VDD | V |
| “L” input current | IIL | CE Pad, VIL=0V |  |  |  | -10 |  | μA |
| Oscillation Disable Time | Tplz | OUT Pad |  |  |  |  | 0.2 | μs |
| Oscillation Enable Time | Tpzl | OUT Pad |  |  |  |  | 2.0 | ms |
| Oscillation Start up Time | Tstart | VDD=1.2V |  |  |  |  | 5.0 | ms |
|  |  | VDD=1.8V, VDD=3.3V |  |  |  |  | 2.0 |  |
| “H” output voltage | VOH | OUT Pad, IOH=-1.0mA, VDD=3.3V |  |  | 0.9VDD |  |  | V |
| “L” output voltage | VOL | OUT Pad, IOL=1.0mA, VDD=3.3V |  |  |  |  | 0.1VDD | V |
| Current consumption | IDD | VDD=1.2V No Load | IPS0503N9JA |  |  |  | TBD | μA |
|  |  |  | IPS0503NAJA |  |  |  | 26.0 |  |
|  |  | VDD=1.8V No Load | IPS0503N9JA |  |  |  | 14.5 |  |
|  |  |  | IPS0503NAJA |  |  |  | 26.0 |  |
|  |  | VDD=3.3V No Load | IPS0503N9JA |  |  |  | 15.0 |  |
|  |  |  | IPS0503NAJA |  |  |  | 27.0 |  |
| Current consumption at oscillation disable | IDDD | VDD=3.3V, CE=GND |  |  |  | 0.6 | 3.0 | μA |
| Freq. VDD deviation | FVST | VDD±10% (VDD=1.2V, 1.8V, 3.3V) |  |  |  |  | ±1.0 | ppm |
| Output Duty Ratio | Duty | CL=15pF, 1/2VDD point |  |  | 45 |  | 55 | % |
| Rise time/Fall time | Tr/Tf | CL=15pF 10%⇔90%VDD |  | VDD=1.2V |  |  | 30.0 | ns |
|  |  |  |  | VDD=1.8V |  |  | 15.0 |  |
|  |  |  |  | VDD=3.3V |  |  | 7.5 |  |

Fig. 5-1 Output wave form (Duty, Tr, Tf, VOH, VOL)

VIH : Threshold voltage for Oscillation Start

VIL : Threshold voltage for Oscillation Stop

Fig. 5-2 Input output signal timing

- Circuit Parameters of Oscillator (Reference Data for Circuit Design)

Ta=25℃

| Parameter | Symbol | Condition | Typical value | Unit |
| --- | --- | --- | --- | --- |
| Regulated Voltage | Vreg |  | 0.8 | V |
| Feedback Resistor | Rf |  | 570 | kΩ |
| Driving Resistor | Rd |  | 1000 | Ω |
| Oscillation Capacitor | Cg | Gate side | 3.0 | pF |
|  | Cd | Drain side | 1.0 | pF |

* The above values are the design values and are not guaranteed by test.

Fig. 6-1 Block Diagram

- Pad Layout

7-1 Cross Type

VSS

X1

CE

VDD

OUT

0.55mm

0.54mm

X2

Scribe line center

Fig. 7-1 Pad Layout of IPS0503NzJAX

7-2 Flip Type

VDD

X1

CE

VSS

OUT

0.55mm

0.54mm

X2

Scribe line center

Fig. 7-2 Pad Layout of IPS0503NzJAF

- Revision History

| Revision No. | Revision Date | Revised items | Before Revision | After Revision |
| --- | --- | --- | --- | --- |
| SC-7.1 | 2025/11/04 | 1st edition | - | - |