- Description

IPS020*E is the IC for differential output SPXO whose Phase Noise is quite low. This IC suites for telecommunication applications. This IC also supports pCML (Pseudo CML) output.

We also offer models with the best pad layout for flip chip bonding.

- Features

- Operation temperature	: -40℃~125℃

- Power supply voltage	: 2.375V~3.63V (LVDS & HCSL)

2.375V~2.625V (2.5V LVPECL)

2.97V~3.63V (3.3V LVPECL & pCML)

- Standby function		: Oscillation stop

- Crystal frequency		: 220MHz maximum

- Output		        : LVPECL / LVDS / HCSL / Pseudo CML (HDLVDS : High Drive LVDS)

- Crystal mode		: Fundamental & 3rd overtone

- Small chip size		: 0.65mm × 0.75mm

- Pad layout                : Straight type & Flip type

- Duty cycle		        : Within 50%±5%

- Phase jitter               : ~57fsec (12kHz to 20MHz at 156.25MHz)

- Part number rule

The contents of this sheet are subject to change without notice.

- Series

| Part Number |  |  |  |  |  |  |  | Output | Crystal Frequency (MHz) |  |  | Divide | Output Frequency (MHz) |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Mode | Min. | Max. |  | Min. | Max. |  |
| IPS020 | 3 | M | 0 | P |  | E | S | LVPECL (VDD= 3.3V ±10%) | Fund. | 27 | 100 | 1/1 | 27 | 100 | Straight type |
| IPS020 | 4 | M | 0 | P |  | E | S |  |  | 80 | 130 |  | 80 | 130 |  |
| IPS020 | 5 | M | 0 | P |  | E | S |  |  | 120 | 170 |  | 120 | 170 |  |
| IPS020 | 6 | M | 0 | P |  | E | S |  |  | 160 | 220 |  | 160 | 220 |  |
| IPS020 | 3 | M | 1 | P |  | E | S |  |  | 27 | 100 | 1/2 | 13.5 | 50 |  |
| IPS020 | 3 | M | T | P |  | E | S |  | 3rd | 65 | 90 | 1/1 | 65 | 90 |  |
| IPS020 | 4 | M | T | P |  | E | S |  |  | 90 | 125 |  | 90 | 125 |  |
| IPS020 | 5 | M | T | P |  | E | S |  |  | 125 | 160 |  | 125 | 160 |  |
| IPS020 | 6 | M | T | P |  | E | S |  |  | 160 | 220 |  | 160 | 220 |  |
| IPS020 | 4 | M | 0 | P |  | E | F |  | Fund. | 50 | 150 | 1/1 | 50 | 150 | Flip type |
| IPS020 | 5 | M | 0 | P |  | E | F |  |  | 100 | 200 |  | 100 | 200 |  |
| IPS020 | 3 | M | 0 | P | A | E | S | LVPECL (VDD= 2.5V ±5%) | Fund. | 27 | 100 | 1/1 | 27 | 100 | Straight type |
| IPS020 | 4 | M | 0 | P | A | E | S |  |  | 80 | 130 |  | 80 | 130 |  |
| IPS020 | 5 | M | 0 | P | A | E | S |  |  | 120 | 170 |  | 120 | 170 |  |
| IPS020 | 6 | M | 0 | P | A | E | S |  |  | 160 | 220 |  | 160 | 220 |  |
| IPS020 | 3 | M | 1 | P | A | E | S |  |  | 27 | 100 | 1/2 | 13.5 | 50 |  |
| IPS020 | 3 | M | T | P | A | E | S |  | 3rd | 65 | 90 | 1/1 | 65 | 90 |  |
| IPS020 | 4 | M | T | P | A | E | S |  |  | 90 | 125 |  | 90 | 125 |  |
| IPS020 | 5 | M | T | P | A | E | S |  |  | 125 | 160 |  | 125 | 160 |  |
| IPS020 | 6 | M | T | P | A | E | S |  |  | 160 | 220 |  | 160 | 220 |  |
| IPS020 | 4 | M | 0 | P | A | E | F |  | Fund. | 50 | 150 | 1/1 | 50 | 150 | Flip type |
| IPS020 | 5 | M | 0 | P | A | E | F |  |  | 100 | 200 |  | 100 | 200 |  |
| IPS020 | 3 | M | 0 | D |  | E | S | LVDS | Fund. | 27 | 100 | 1/1 | 27 | 100 | Straight type |
| IPS020 | 4 | M | 0 | D |  | E | S |  |  | 80 | 130 |  | 80 | 130 |  |
| IPS020 | 5 | M | 0 | D |  | E | S |  |  | 120 | 170 |  | 120 | 170 |  |
| IPS020 | 6 | M | 0 | D |  | E | S |  |  | 160 | 220 |  | 160 | 220 |  |
| IPS020 | 3 | M | 1 | D |  | E | S |  |  | 27 | 100 | 1/2 | 13.5 | 50 |  |
| IPS020 | 3 | M | T | D |  | E | S |  | 3rd | 65 | 90 | 1/1 | 65 | 90 |  |
| IPS020 | 4 | M | T | D |  | E | S |  |  | 90 | 125 |  | 90 | 125 |  |
| IPS020 | 5 | M | T | D |  | E | S |  |  | 125 | 160 |  | 125 | 160 |  |
| IPS020 | 6 | M | T | D |  | E | S |  |  | 160 | 220 |  | 160 | 220 |  |
| IPS020 | 4 | M | 0 | D |  | E | F |  | Fund. | 50 | 150 | 1/1 | 50 | 150 | Flip type |
| IPS020 | 5 | M | 0 | D |  | E | F |  |  | 100 | 200 |  | 100 | 200 |  |

| Part Number |  |  |  |  |  |  |  | Output | Crystal Frequency (MHz) |  |  | Divide | Output Frequency (MHz) |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Mode | Min. | Max. |  | Min. | Max. |  |
| IPS020 | 3 | M | 0 | H |  | E | S | HCSL | Fund. | 27 | 100 | 1/1 | 27 | 100 | Straight type |
| IPS020 | 4 | M | 0 | H |  | E | S |  |  | 80 | 130 |  | 80 | 130 |  |
| IPS020 | 5 | M | 0 | H |  | E | S |  |  | 120 | 170 |  | 120 | 170 |  |
| IPS020 | 6 | M | 0 | H |  | E | S |  |  | 160 | 220 |  | 160 | 220 |  |
| IPS020 | 3 | M | 1 | H |  | E | S |  |  | 27 | 100 | 1/2 | 13.5 | 50 |  |
| IPS020 | 3 | M | T | H |  | E | S |  | 3rd | 65 | 90 | 1/1 | 65 | 90 |  |
| IPS020 | 4 | M | T | H |  | E | S |  |  | 90 | 125 |  | 90 | 125 |  |
| IPS020 | 5 | M | T | H |  | E | S |  |  | 125 | 160 |  | 125 | 160 |  |
| IPS020 | 6 | M | T | H |  | E | S |  |  | 160 | 220 |  | 160 | 220 |  |
| IPS020 | 4 | M | 0 | H |  | E | F |  | Fund. | 80 | 130 | 1/1 | 80 | 130 | Flip type |
| IPS020 | 5 | M | 0 | H |  | E | F |  |  | 120 | 170 |  | 120 | 170 |  |
| IPS020 | 5 | M | 0 | C |  | E | F | pCML | Fund. | 100 | 200 | 1/1 | 100 | 200 | Flip type |
| IPS020 | 4 | M | T | C |  | E | S |  | 3rd | 90 | 125 | 1/1 | 90 | 125 | Straight type |
| IPS020 | 5 | M | T | C |  | E | S |  |  | 125 | 160 |  | 125 | 160 |  |

- Absolute Maximum Ratings

VSS=0V, Ta=25℃± 2℃

| Parameter | Symbol | Condition | Ratings |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max | Unit |
| Supply Voltage | VDD |  | VSS-0.5 | 5.0 | V |
| Input Voltage | VIN | All Input Pin | VSS-0.5 | VDD+0.5 | V |
| Output Voltage | VOUT |  | VSS-0.5 | VDD+0.5 | V |
| Output Current | IOUT |  |  | 25 | mA |
| Junction Temperature | Tj |  | -55 | 150 | ℃ |
| Storage Temperature | Tstg |  | -55 | 125 | ℃ |

- Recommended Operating Condition

VSS=0V, Ta=-40℃~125℃

| Parameter | Symbol | Condition | Min | Typ | Max | Unit | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Supply Voltage | VDD | 2.5V LVPECL | 2.375 | 2.50 | 2.625 | V | VDD |
|  |  | LVDS, HCSL | 2.375 | 3.30 | 3.63 |  |  |
|  |  | 3.3V LVPECL, pCML | 2.97 | 3.30 | 3.63 |  |  |
| “H” Input Voltage | VIH |  | VDD×0.7 |  |  | V | CE |
| “L” Input Voltage | VIL |  |  |  | VDD×0.3 | V | CE |
| Input Voltage | VIN |  | VSS |  | VDD | V | CE |
| Output Load Resistance | RL | LVPECL ※1 | 49.5 | 50.0 | 50.5 | Ω | OUT |
|  |  | LVDS ※2 | 99 | 100 | 101 |  |  |
|  |  | HCSL ※3 | 49.5 | 50.0 | 50.5 |  |  |
|  |  | pCML ※4 | 80 | 100 | 120 |  |  |
| Ambient Temperature | Topt |  | -40 |  | 125 | ℃ |  |

This IC has enough immunity against ESD and Latch-up, but handle with care.

※1 Terminate to VDD-2.0V

※2 Between OUT and OUTN

※3 To GND and Rs=33Ω or 0Ω

※4 Refer to Fig.4-1

Fig. 4-1 pCML RL condition

- Electrical Specification

5-1 LVPECL Output (VDD=3.3V Operation)

5-1-1 DC Characteristics

Unless otherwise stated, VDD=2.97V~3.63V, VSS=0V, Ta=-40℃~125℃

| Parameter | Symbol | Condition |  | Specification |  |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Typ | Max |  |
| Output leak current | IZ | CE=0.3V |  |  |  | 10 | μA |
| “L” input current | IIL | VIN=VSS |  |  | -10 |  | μA |
| “H” output voltage | VOH | RL=50Ω(Terminated to VDD-2.0V) CE=Open, OUT/OUTN |  | VDD -1.025 | VDD -0.950 | VDD -0.880 | V |
| “L” output voltage | VOL |  |  | VDD -1.810 | VDD -1.700 | VDD -1.620 | V |
| Current consumption※ | IDD | IPS0203M0PES, F0=100MHz |  |  | 34.0 | 51.0 | mA |
|  |  | IPS0204M0PEq, F0=122.88MHz |  |  | 36.0 | 54.0 |  |
|  |  | IPS0205M0PEq, F0=170MHz |  |  | 38.0 | 57.0 |  |
|  |  | IPS0206M0PES, F0=212.5MHz |  |  | 38.0 | 57.0 |  |
|  |  | IPS0203MTPES, F0=90MHz |  |  | 35.0 | 52.0 |  |
|  |  | IPS0204MTPES, F0=125MHz |  |  | 37.0 | 55.0 |  |
|  |  | IPS0205MTPES, F0=156.25MHz |  |  | 38.0 | 57.0 |  |
|  |  | IPS0206MTPES, F0=200MHz |  |  | 40.0 | 60.0 |  |
| Current consumption at oscillation stop | IDDD | CE=GND | Straight type |  |  | 10 | uA |
|  |  |  | Flip type |  | 60 | 300 |  |

※Condition : VDD=3.3V, CE=Open, RL=50Ω (Terminated to VDD-2.0V)

5-1-2 Switching Characteristics

Unless otherwise stated, VDD=2.97V~3.63V, VSS=0V, Ta=-40℃~125℃

| Parameter | Symbol | Condition |  | Specification |  |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Typ | Max |  |
| Oscillation start up time | Tstart | IPS020xM0 |  |  |  | 2.0 | ms |
|  |  | IPS020xMT |  |  |  | 10.0 |  |
| Output Disable Time | Tplz |  |  |  |  | 200 | ns |
| Output Enable Time | Tpzl | IPS020xM0 |  |  |  | 2.0 | ms |
|  |  | IPS020xMT |  |  |  | 10.0 |  |
| Rise time / Fall time | Tr / Tf | 20%~80% Vopp |  |  | 0.25 | 0.40 | ns |
| Output Duty Ratio | Duty | 1/2Vopp point |  | 45 |  | 55 | % |
|  |  | Only IPS0206MT 1/2Vopp point | Ta=-40℃~85℃ | 45 |  | 55 |  |
|  |  |  | Ta=85℃~125℃ | 40 |  | 60 |  |
| Output Swing | Vopp |  |  | 0.4 |  |  | V |
| Freq. VDD deviation | Fvst | VDD=3.3V±10% |  |  |  | ±2.0 | ppm |

5-2 LVPECL Output (VDD=2.5V Operation)

5-2-1 DC Characteristics

Unless otherwise stated, VDD=2.375V~2.625V, VSS=0V, Ta=-40℃~125℃

| Parameter | Symbol | Condition |  | Specification |  |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Typ | Max |  |
| Output leak current | IZ | CE=0.3V |  |  |  | 10 | μA |
| “L” input current | IIL | VIN=VSS |  |  | -10 |  | μA |
| “H” output voltage | VOH | RL=50Ω(Terminated to VDD-2.0V) CE=Open, OUT/OUTN |  | VDD -1.105 | VDD -0.950 | VDD -0.880 | V |
| “L” output voltage | VOL |  |  | VDD -1.810 | VDD -1.700 | VDD -1.620 | V |
| Current consumption※ | IDD | IPS0203M0PAES, F0=100MHz |  |  | 31 | 46 | mA |
|  |  | IPS0204M0PAEq, F0=122.88MHz |  |  | 33 | 50 |  |
|  |  | IPS0205M0PAEq, F0=170MHz |  |  | 33 | 50 |  |
|  |  | IPS0206M0PAES, F0=212.5MHz |  |  | 33 | 50 |  |
|  |  | IPS0203MTPAES, F0=90MHz |  |  | 32 | 48 |  |
|  |  | IPS0204MTPAES, F0=125MHz |  |  | 34 | 51 |  |
|  |  | IPS0205MTPAES, F0=156.25MHz |  |  | 35 | 53 |  |
|  |  | IPS0206MTPAES, F0=200MHz |  |  | 37 | 56 |  |
| Current consumption at oscillation stop | IDDD | CE=GND | Straight type |  |  | 10 | uA |
|  |  |  | Flip type |  | 60 | 300 |  |

※Condition : VDD=2.5V, CE=Open, RL=50Ω (Terminated to VDD-2.0V)

5-2-2 Switching Characteristics

Unless otherwise stated, VDD=2.375V~2.625V, VSS=0V, Ta=-40℃~125℃

| Parameter | Symbol | Condition |  | Specification |  |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Typ | Max |  |
| Oscillation start up time | Tstart | IPS020xM0 |  |  |  | 2.0 | ms |
|  |  | IPS020xMT |  |  |  | 10 |  |
| Output Disable Time | Tplz |  |  |  |  | 200 | ns |
| Output Enable Time | Tpzl | IPS020xM0 |  |  |  | 2.0 | ms |
|  |  | IPS020xMT |  |  |  | 10 |  |
| Rise time / Fall time | Tr / Tf | 20%~80% Vopp |  |  | 0.25 | 0.40 | ns |
| Output Duty Ratio | Duty | 1/2Vopp point |  | 45 |  | 55 | % |
|  |  | Only IPS0206MT 1/2Vopp point | Ta=-40℃~85℃ | 45 |  | 55 |  |
|  |  |  | Ta=85℃~125℃ | 40 |  | 60 |  |
| Output Swing | Vopp |  |  | 0.4 |  |  | V |
| Freq. VDD deviation | Fvst | VDD=2.5V±5% |  |  |  | ±2.0 | ppm |

5-3 LVDS Output

5-3-1 DC Characteristics

Unless otherwise stated, VDD=2.375V~3.63V, VSS=0V, Ta=-40℃~125℃

| Parameter | Symbol | Condition |  | Specification |  |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Typ | Max |  |
| Output leak current | IZ | CE=0.3V |  |  |  | 10 | μA |
| “L” input current | IIL | VIN=VSS |  |  | -10 |  | μA |
| “H” output voltage | VOH | RL=100Ω(Between OUT & OUTN) CE=Open, OUT/OUTN |  |  | 1.43 | 1.60 | V |
| “L” output voltage | VOL |  |  | 0.90 | 1.10 |  | V |
| Differential voltage | VOD |  |  | 247 | 330 | 454 | mV |
| Differential voltage deviation | ⊿VOD |  |  |  |  | 50 | mV |
| Offset voltage | VOS |  |  | 1.125 | 1.250 | 1.375 | V |
| Offset voltage deviation | ⊿VOS |  |  |  |  | 50 | mV |
| Current consumption※ | IDD | IPS0203M0DES, F0=100MHz |  |  | 8.5 | 17.0 | mA |
|  |  | IPS0204M0DES, F0=122.88MHz |  |  | 10.5 | 21.0 |  |
|  |  | IPS0205M0DES, F0=170MHz |  |  | 12.5 | 25.0 |  |
|  |  | IPS0206M0DES, F0=212.5MHz |  |  | 12.5 | 25.0 |  |
|  |  | IPS0203MTDES, F0=90MHz |  |  | 10.5 | 21.0 |  |
|  |  | IPS0204MTDES, F0=125MHz |  |  | 14.0 | 28.0 |  |
|  |  | IPS0205MTDES, F0=156.25MHz |  |  | 15.0 | 30.0 |  |
|  |  | IPS0206MTDES, F0=200MHz |  |  | 17.0 | 34.0 |  |
|  |  | IPS0204M0DEF, F0=122.88MHz |  |  | 12.0 | 24.0 |  |
|  |  | IPS0205M0DEF, F0=170MHz |  |  | 13.0 | 26.0 |  |
| Current consumption at oscillation stop | IDDD | CE=GND | Straight type |  |  | 10 | uA |
|  |  |  | Flip type |  | 60 | 300 |  |

※Condition : VDD=3.3V, CE=Open, RL=100Ω (Between OUT & OUTN)

5-3-2 Switching Characteristics

Unless otherwise stated, VDD=2.375V~3.63V, VSS=0V, Ta=-40℃~125℃

| Parameter | Symbol | Condition |  | Specification |  |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Typ | Max |  |
| Oscillation start up time | Tstart | IPS020xM0, IPS0203M1 |  |  |  | 2.0 | ms |
|  |  | IPS020xMT |  |  |  | 10 |  |
| Output Disable Time | Tplz |  |  |  |  | 200 | ns |
| Output Enable Time | Tpzl | IPS020xM0, IPS0203M1 |  |  |  | 2.0 | ms |
|  |  | IPS020xMT |  |  |  | 10 |  |
| Rise time / Fall time | Tr / Tf | 20%~80% Vopp |  |  |  | 0.40 | ns |
| Output Duty Ratio | Duty | 1/2Vopp point |  | 45 |  | 55 | % |
|  |  | Only IPS0206MT 1/2Vopp point | Ta=-40℃~85℃ | 45 |  | 55 |  |
|  |  |  | Ta=85℃~125℃ | 40 |  | 60 |  |
| Output Swing | Vopp |  |  | 0.25 |  |  | V |
| Freq. VDD deviation | Fvst | VDD=3.3V±10% |  |  |  | ±2.0 | ppm |
|  |  | VDD=2.5V±5% |  |  |  | ±2.0 |  |

5-4 HCSL Output

5-4-1 DC Characteristics

Unless otherwise stated, VDD=2.375V~3.63V, VSS=0V, Ta=-40℃~125℃

| Parameter | Symbol | Condition | Specification |  |  | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Typ | Max |  |
| Output leak current | IZ | CE=0.3V |  |  | 10 | μA |
| “L” input current | IIL | VIN=VSS |  | -10 |  | μA |
| “H” output voltage | VOH | RL=50Ω(To GND), Rs=0Ω CE=Open, OUT/OUTN | 550 |  | 900 | mV |
| “L” output voltage | VOL |  | -150 |  | 150 | mV |
| Current consumption※ | IDD | IPS0203M0HES, F0=100MHz |  | 20.0 | 30.0 | mA |
|  |  | IPS0204M0HEq, F0=122.88MHz |  | 22.0 | 33.0 |  |
|  |  | IPS0205M0HEq, F0=170MHz |  | 23.0 | 35.0 |  |
|  |  | IPS0206M0HES, F0=212.5MHz |  | 23.0 | 35.0 |  |
|  |  | IPS0203MTHES, F0=90MHz |  | 22.0 | 33.0 |  |
|  |  | IPS0204MTHES, F0=125MHz |  | 24.0 | 36.0 |  |
|  |  | IPS0205MTHES, F0=156.25MHz |  | 24.0 | 36.0 |  |
|  |  | IPS0206MTHES, F0=200MHz |  | 27.0 | 38.0 |  |
| Current consumption at oscillation stop | IDDD | CE=GND |  |  | 10 | uA |

※Condition : VDD=3.3V, CE=Open, RL=50Ω(To GND), Rs=0Ω

5-4-2 Switching Characteristics

Unless otherwise stated, VDD=2.375V~3.63V, VSS=0V, Ta=-40℃~125℃

| Parameter | Symbol | Condition |  | Specification |  |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Typ | Max |  |
| Oscillation start up time | Tstart | IPS020xM0 |  |  |  | 2.0 | ms |
|  |  | IPS020xMT |  |  |  | 10 |  |
| Output Disable Time | Tplz |  |  |  |  | 200 | ns |
| Output Enable Time | Tpzl | IPS020xM0 |  |  |  | 2.0 | ms |
|  |  | IPS020xMT |  |  |  | 10 |  |
| Rise time / Fall time | Tr / Tf | 20%~80% Vopp, Rs=0Ω |  |  | 0.30 | 0.50 | ns |
| Output Duty Ratio | Duty | 1/2Vopp point |  | 45 |  | 55 | % |
|  |  | Only IPS0206MT 1/2Vopp point | Ta=-40℃~85℃ | 45 |  | 55 |  |
|  |  |  | Ta=85℃~125℃ | 40 |  | 60 |  |
| Output Swing | Vopp | Rs=0Ω |  | 0.55 |  |  | V |
| Freq. VDD deviation | Fvst | VDD=3.3V±10% |  |  |  | ±2.0 | ppm |
|  |  | VDD=2.5V±5% |  |  |  | ±2.0 |  |

5-5 pCML Output

5-5-1 DC Characteristics

Unless otherwise stated, VDD=2.97V~3.63V, VSS=0V, Ta=-40℃~125℃

| Parameter | Symbol | Condition | Specification |  |  | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Typ | Max |  |
| Output leak current | IZ | CE=0.3V |  |  | 10 | μA |
| “L” input current | IIL | VIN=VSS |  | -10 |  | μA |
| Current consumption※ | IDD | IPS0205M0CEF, F0=170MHz |  | 16.5 | 33.0 | mA |
|  |  | IPS0204MTCES, F0=125MHz |  | 17.5 | 35.0 |  |
|  |  | IPS0205MTCES, F0=156.25MHz |  | 18.0 | 36.0 |  |
| Current consumption at oscillation stop | IDDD | CE=GND |  | 100 | 300 | uA |

※Condition : VDD=3.3V, CE=Open, RL=100Ω(AC coupling)

5-5-2 Switching Characteristics

Unless otherwise stated, VDD=2.97V~3.63V, VSS=0V, Ta=-40℃~125℃

| Parameter | Symbol | Condition | Specification |  |  | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Typ | Max |  |
| Oscillation start up time | Tstart | IPS020xM0 |  |  | 2.0 | ms |
|  |  | IPS020xMT |  |  | 10 |  |
| Output Disable Time | Tplz |  |  |  | 200 | ns |
| Output Enable Time | Tpzl | IPS020xM0 |  |  | 2.0 | ms |
|  |  | IPS020xMT |  |  | 10 |  |
| Rise time / Fall time | Tr / Tf | 20%~80% Vopp, Rs=0Ω |  | 0.25 | 0.35 | ns |
| Output Duty Ratio | Duty | 1/2Vopp point | 45 |  | 55 | % |
| Output Swing | Vopp |  | 0.6 |  |  | V |
| Freq. VDD deviation | Fvst | VDD=3.3V±10% |  |  | ±2.0 | ppm |

Fig. 5-1　Output Wave Form (Duty, Tr, Tf, VOH, VOL, Vopp) of LVPEL & HCSL

Fig. 5-2　Output Wave Form (Duty, Tr, Tf, Vopp) of pCML

Fig. 5-3　Output Wave Form (Duty, Tr, Tf, VOH, VOL,VOD,VOS, Vopp) of LVDS

VIH : Threshold voltage for Oscillation Start

VIL : Threshold voltage for Oscillation Stop

Fig. 5-4　Input output signal timing

- Circuit Parameters of Oscillator (Reference Data for Circuit Design)

6-1 Feedback Resister

Ta=25℃

| Parameter | Symbol | Condition | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| IPS020xM0, IPS0203M1 | Rf |  |  | 200 |  | kΩ |
| IPS0203MT |  |  |  | 4.00 |  |  |
| IPS0204MT |  |  |  | 3.00 |  |  |
| IPS0205MT |  | Only pCML |  | 2.00 |  |  |
|  |  | Others |  | 1.75 |  |  |
| IPS0206MT |  |  |  | 0.75 |  |  |

*The above values are the design values and are not guaranteed by test.

6-2 Driving Resister

Ta=25℃

| Parameter | Symbol | Condition | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| IPS0203M0, IPS0203M1 | Rd |  |  | 1000 |  | Ω |
| IPS0204M0 |  | Flip type |  | 757 |  |  |
|  |  | Straight type |  | 800 |  |  |
| IPS0205M0 |  | Flip type |  | 400 |  |  |
|  |  | Straight type |  | 500 |  |  |
| IPS0206M0 |  |  |  | 300 |  |  |
| IPS0203MT |  |  |  | 500 |  |  |
| IPS0204MT |  | Only pCML |  | 600 |  |  |
|  |  | Others |  | 400 |  |  |
| IPS0205MT |  | Only pCML |  | 400 |  |  |
|  |  | Others |  | 200 |  |  |
| IPS0206MT |  |  |  | 200 |  |  |

*The above values are the design values and are not guaranteed by test.

6-3 Oscillation Capacitor

Ta=25℃

| Parameter | Symbol | Condition | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| IPS0203M0 IPS0203M1 | Cg |  |  | 6.0 |  | pF |
|  | Cd |  |  | 9.0 |  |  |
| IPS0204M0 | Cg | Flip type |  | 10.0 |  |  |
|  |  | Straight type |  | 5.0 |  |  |
|  | Cd | Flip type |  | 10.0 |  |  |
|  |  | Straight type |  | 8.0 |  |  |
| IPS0205M0 | Cg | Flip type |  | 10.0 |  |  |
|  |  | Straight type |  | 5.0 |  |  |
|  | Cd | Flip type |  | 10.0 |  |  |
|  |  | Straight type |  | 7.0 |  |  |
| IPS0206M0 | Cg |  |  | 4.0 |  |  |
|  | Cd |  |  | 6.0 |  |  |
| IPS0203MT | Cg |  |  | 7.0 |  |  |
|  | Cd |  |  | 9.0 |  |  |
| IPS0204MT | Cg | Only pCML |  | 8.0 |  |  |
|  |  | Others |  | 6.0 |  |  |
|  | Cd | Only pCML |  | 10.0 |  |  |
|  |  | Others |  | 8.0 |  |  |
| IPS0205MT | Cg | Only pCML |  | 8.0 |  |  |
|  |  | Others |  | 6.0 |  |  |
|  | Cd | Only pCML |  | 10.0 |  |  |
|  |  | Others |  | 8.0 |  |  |
| IPS0206MT | Cg |  |  | 3.0 |  |  |
|  | Cd |  |  | 8.0 |  |  |

*The above values are the design values and are not guaranteed by test.

※

        - Output buffer according to each output waveform format

Fig. 6-1　Block Diagram

- Pad Layout

7-1 Straight Type

0.65mm

VDD2

VDD

y

OUT

X1

0.75mm

x

OUTN

X2

VSS

CE

Scribe line center

7-2 Flip Type

0.65mm

VSS

CE

y

OUT

X1

0.75mm

x

VDD2

X2

VDD

OUTN

Scribe line center

IC Identification Code (Option)

Fuse  □□

Code  9 10

IC Part # Identification

□ : Fuse no cut ■ : Fuse cut

IC Identification Code

Fuse  □□□　□□□　□□

Code  1 2 3  4 5 6  7 8

LOGO : IPS0208_M_F (Flip type)

IPS0208M_S3 (Straight type of pCML)

IPS020_M_S3 (Others)

- Revision History

| Revision No. | Revision Date | Revised items | Contents |  |
| --- | --- | --- | --- | --- |
|  |  |  | Before Revision | After Revision |
| SD-2.3 | 2025/11/04 | Wafer thickness 100um | Listed | As requested |
|  |  | IPS0203M1PES | Under Development | Listed |
|  |  | IPS0204M0HEF | Unlisted | Listed |
|  |  | IPS0205M0HEF | Unlisted | Listed |
|  |  | IPS0203M1HES | Under Development | Listed |
|  |  | IDDD Condition | CE≦0.3V | CE=GND |